COMPUTER GRAPHICS GROUP WORK

(a)Architectural floor plan-10

(b)Electrical wiring plan-6

(c)Water & plumbing plan-6

(d)Wi-Fi & network plan-5

(e)Presentation & professionalism-3

Total=30

NOTES FOR STUDENTS

- You may design one typical floor and clearly indicate that it applies to all 14 floors.

- Use standard architectural and engineering symbols.

- Ensure clarity, accuracy, and realism in your design.

INSTRUCTIONS TO CANDIDATES

- This is a practical examination.

- You are required to use AutoCAD or Archicad.

- All drawings must be neat, properly scaled, and fully labeled.

- Save your work using the format: REGNO_COMPUTERGRAPHICS_CAT.dwg / .pln

- Share your work in GIT HUB and provide the link in the portal

- Submit both:

- The editable file

- A PDF export of your drawings

QUESTION (30 MARKS)

You are required to design and draw an architectural plan for a 14-floor commercial building using AutoCAD or Archicad. The design should demonstrate correct application of computer graphics principles, technical accuracy, and clarity.

Your submission must include the following components:

(a) Building Floor Plan Design (10 Marks)

Create a typical floor plan for one floor of the 14-storey building showing:

- Walls, doors, windows, staircases, and elevator shafts

- Proper room layout (offices, corridors, washrooms)

- Correct use of layers and line types

- Clear dimensions and labels

(b) Electrical Wiring Layout (6 Marks)

Design the electrical wiring plan for the building showing:

- Main power supply line

- Distribution panels

- Lighting points and socket outlets

- Switch positions

- Appropriate electrical symbols and legends

(c) Water Supply and Plumbing Plan (6 Marks)

Draw the water and plumbing system indicating:

- Main water inlet

- Vertical risers and horizontal distribution pipes

- Washroom connections

- Water tanks or pump rooms

- Correct plumbing symbols and pipe routing

(d) Wi-Fi and Network Cabling Plan (5 Marks)

Design the Wi-Fi and network wiring layout showing:

- Network backbone

- Distribution switches per floor

- Wi-Fi access point locations

- Data cabling routes

- Proper labeling and symbols

(e) Presentation and Professionalism (3 Marks)

Marks will be awarded for:

- Proper use of layers and naming conventions

- Drawing organization and neatness

- Clear legends, titles, and scale

- Correct file submission format

mutio18/COMPUTERGRAPHICS.CAT

https://github.com/mutio18/COMPUTERGRAPHICS.CAT

DOCUMENTATION

Below are the tables of each separate layer and their elements.

| Layer Name | colour | line weight | objects |
| --- | --- | --- | --- |
| A-WALL | Yellow | 0.30mm | Outer walls, Inner walls and partitions |
| A-DOOR | Green | 0.25mm | door blocks |
| A-WIND | Cyan | 0.25mm | Windows |
| A-STAIR | Magenta | 0.25mm | Stairs, arrows |
| A-ELEV | Blue | 0.25mm | Elevator shafts, doors |
| A-CORE | Red | 0.25mm | Core rectangle |
| A-CLOR | gray | 0.18mm | Corridors & pathways |
| A-ANNO-TEXT | white | 0.18mm | Room labels |
| A-ANNO-DIMS | white | 0.18mm | Dimensions |

| Layer name | colour | Line weight | Objects |
| --- | --- | --- | --- |
| E-POWER-MAIN | Red | 0.30mm | Main power supply line |
| E-PANELS | Magenta | 0.25mm | Distribution panel |
| E-LIGHTING | Yellow | 0.25mm | Light fixtures |
| E-SWITCHING | Green | 0.25mm | Switches |
| E-OUTLETS | Blue | 0.25mm | Socket outlets |
| E-CONDUIT | Cyan | 0.18mm(dashed) | Switch- lights line |
| E-CONDUIT-POWER | Red | 0.25mm | Panel-switch lines |
| E-CONDUIT-SOCKET | Green | 0.25mm | Panel- socket lines |
| E-LABELS | White | 0.18mm | Electrical legends |

| Layer name | colour | Line weight | Objects |
| --- | --- | --- | --- |
| P-WATER-MAIN | Blue | 0.30mm | Main water inlet |
| P-COLD-WATER | Cyan | 0.25mm | Cold water pipes |
| P-HOT-WATER | Red | 0.25mm | Hot water pipes |
| P-DRAIN | Brown | 0.25mm(dashed) | Drainage pipes |
| P-FIXTURES | Magenta | 0.18mm | Toilet, sinks, urinals & drains |
| P-RISERS | Blue | 0.30mm center | Water riser |
| P-LABELS | White | 0.18mm | Legend |

| Layer name | Colour | Line weight | Objects |
| --- | --- | --- | --- |
| N-BACKBONE | Red | 0.30mm | Fiber backbone line |
| N-SWITCHES | Magenta | 0.25mm | Switch |
| N-AP | Green | 0.25mm | Wi-Fi access points |
| N-CABLING | Yellow | 0.20mm(dashed) | CAT6 lines to outlets |
| N-OUTLETS | Blue | 0.20mm | Data outlets |
| N-LABELS | White | 0.18mm | Network legends |

Architectural Floor Plan

A typical floor measuring 30 meters by 40 meters was developed to represent all 14 floors of the commercial building. The layout features a central core containing two staircases for fire safety and a bank of three elevators for circulation. The core is surrounded by a 1.8 meter wide corridor that provides access to all spaces. The bottom-left corner has male, female, and accessible washrooms while the right side contains a break room, copy room, server room, storage and janitorial closet. The top and left zones are dedicated to office spaces, with two conference rooms positioned at the top corners. All rooms are properly labeled and connected to the corridor network.

Electrical Wiring Plan

The electrical wiring plan illustrates the complete power distribution system for the typical floor. A main distribution panel (DP-01) was placed in the server room to serve as the central power distribution point. Light fixtures were strategically placed in all rooms, with corresponding switches positioned near doorways for convenient operation. Socket outlets were located along walls in offices, conference rooms and common areas to support various electrical devices. The wiring was organized using a color-coded layer system: red lines represent power from panel to switches, green lines show panel-to-socket connections, and blue lines indicate switch-to-light wiring. A main power supply line was drawn from the building entrance to the distribution panel, completing the electrical distribution network.

Water Supply and Plumbing Plan

The plumbing plan details the water supply and drainage systems for the typical floor. A main water inlet was drawn from the building exterior to a water riser (WR-01) located near the washrooms. Cold water supply lines in cyan extend from the riser to all plumbing fixtures, including toilets, sinks, and urinals. Hot water lines in red were added to serve the sink locations. Drainage pipes in brown dashed lines connect each fixture to the main drain stack (DS-01), with appropriate slope indications for proper wastewater flow. All plumbing fixtures were placed as reusable blocks to maintain consistency throughout the design.

Wi-Fi and Network Cabling Plan

The network plan outlines the data communication infrastructure for the floor. A server room serves as the main distribution point connected via a fiber backbone to the floor switch (SW-01). Wi-Fi access points were strategically placed throughout office zones, conference rooms and corridors to ensure comprehensive wireless coverage. Data outlets were positioned along walls in all workspaces to provide wired network connectivity. All access points and outlets are connected to the main switch using a daisy-chain method represented by yellow dashed lines demonstrating the logical cabling pathways while maintaining drawing clarity.